[image: image1.jpg]SAINT
PAUL






November 23, 2010

MARK CEMENSKY

2343 SWAN DRIVE

MENDOTA HEIGHTS MN  55120-1424

Re:
274 Sidney St E 

File#:
10 920794 VB2

Dear Property Owner:

The following is the Code Compliance report you requested on November 16, 2010.

Please be advised that this report is accurate and correct as of the date November 23, 2010.  All deficiencies identified by the City after this date must also be corrected and all codes and ordinances must be complied with.  This report is valid for 365 days from November 23, 2010.  This report may be used in lieu of a Truth in Housing Report required in St Paul Legislative Code 189.  This building must be properly secured and the property maintained at all times.

In order to sell or reoccupy this property the following deficiencies must be corrected:

BUILDING
Inspector:  Jim Seeger
Phone:  651-266-9046

· Insure basement cellar floor is even, is cleanable, and all holes are filled.

· Tuck Point interior/exterior of foundation as necessary.

· Dry out basement and eliminate source of moisture.

· Remove mold, mildew and moldy or water damaged materials.

· Cover water meter pit with concrete or decay resistant, screwed-down cover.  Cleanouts to be flush with floor slab.

· Provide adequate access, ventilation and clearance in crawl space area.

· Install handrails (34 inches - 38 inches above each nosing) and guardrails (36 inch minimum) at all stairways, and return hand rail ends into a newel post or wall per attachment.

· Strap or support top of stair stringers for structural stability.

· Repair or Replace any deteriorated window sash, broken glass, sash holders, re-putty, etc as necessary.

· Provide complete storms and screens, in good repair for all door and window openings.

· Provide functional hardware at all doors and windows

· Repair or replace damaged doors and frames as necessary, including storm doors.

· Weather seal exterior doors, threshold and weather-stripping.

· Repair walls, ceiling and floors throughout, as necessary.

November 23, 2010

274 Sidney St E

Page 2

BUILDING
Inspector:  Jim Seeger
Phone:  651-266-9046

· Air-seal and insulate attic/access door.

· Install Smoke Detectors/Carbon Monoxide Detectors per MN Co Conservation Code and the MN Dept. of Labor and Industry.

· Provide major clean-up of premises.

· Repair siding, soffit, fascia, trim, etc. as necessary.

· Provide proper drainage around house to direct water away from foundation of house.

· Provide proper drainage around house to direct water away from foundation of garage.

· Install rain leaders to direct drainage away from foundation.

· Install flashing in an approved manner at the intersection of the roof with walls, chimneys, and other conjoined surfaces.

· Provide general rehabilitation of garage.

· Install address numbers visible from street and on the alley side of garage.

· Provide durable, dustless parking surface as specified in the zoning code.

· Provide ground cover capable of controlling sediment and erosion.

· Review all applicable codes & policies when replacing windows including egress windows for sleeping rooms.

· Remove trees which are against foundation of home and garage.

· Openings in stair risers must be less than 4 inches.

· Redo basement stairs to code (call Inspector).

· Stabilize foundation on front of house and insure footings are supported. Supply engineering specifications.

· Install tempered glass in windows in rear stairs to code.

· Tuck point rear chimney on house.

· Replace steps at street and front of house.

· A building permit is required to correct the above deficiencies.

ELECTRICAL
Inspector:  Dan Moynihan
Phone:  651-266-9036

· Ground the electrical service to the water service with a copper conductor within 5 feet of the entrance point of the water service

· Bond around water meter with a copper wire sized for the electrical service per Article 250 of the NEC

· Install globe-type enclosed light fixture on all closet lights

· Repair or Replace all broken, missing or loose light fixtures, switches and outlets, covers and plates

· Install hard-wired, battery backup smoke detector per bulletin 80-1 and other smoke detectors as required by the IRC.  Also, Install carbon monoxide detector(s) within 10 feet of all bedrooms

· Physically protect NM cable under range in kitchen.

· All added receptacles must be grounded, tamper-resistant and be on an Arc-Fault Circuit Interrupter-protected circuit.

· Any open walls or walls that are opened as part of this project must be wired to the standards of the 2008 NEC.

November 23, 2010

274 Sidney St E

Page 3

ELECTRICAL
Inspector:  Dan Moynihan
Phone:  651-266-9036

· All buildings on the property must meet the St. Paul Property Maintenance Code (Bulletin 80-1).

· All electrical work must be done by a Minnesota-licensed electrical contractor under an electrical permit.

PLUMBING
Inspector:  Denny Watters
Phone:  651-266-9051

· Basement -  Gas Piping - dryer gas shutoff; connector or piping incorrect, strap to wall (IFGC 402.1)

· Basement -  Gas Piping - run dryer vent to code (IFGC 613.1 - IMC 604.1)

· Basement - Soil and Waste Piping - No permits taken for new plumbing waste vest or water enclosed in walls. Open walls to show all plumbing.

· Basement -  Soil and Waste Piping - unplugged or open piping; back pitched piping (MPC 1000)

· Basement -  Water Meter - raise meter to a minimum 12 inches above floor (MPC 2280)

· Exterior -  Lawn Hydrants - Requires backflow assembly or device (MPC 2000)

· Obtain plumbing permits prior to commencement of work.

HEATING
Inspector:  Gary Stevens
Phone:  651-266-9045

· Install approved lever handle manual gas shutoff valve on all gas appliances and remove unapproved valves.

· Clean and Orsat test wall furnaces burner.  Check all controls for proper operation.  Check furnace heat exchanger for leaks; provide documentation from a licensed contractor that the heating unit is safe.

· Install approved metal chimney liner.

· Replace wall furnace flue venting to code.

· Connect wall furnace venting into chimney liner.

· Provide adequate clearance from flue vent pipe on furnace/boiler to combustible materials or provide approved shielding according to code.

· Vent clothes dryer to code.

· Provide support for gas lines to code.

· Plug, cap and/or remove all disconnected gas lines.

· Bathroom exhaust system to be vented to outside with approved material according to code.  A mechanical ventilation permit is required.

· Provide heat in every habitable room and bathrooms-kitchen. Submit square footage of rooms with electric baseboard heaters and size of heaters.

· A mechanical gas permit is required for the above work.

ZONING

1.  This property is in a(n) R4 zoning district.

2.  This property was inspected as a Single Family Dwelling.

November 23, 2010

274 Sidney St E

Page 4

Notes:

· See attachment for permit requirements and appeals procedure.

· Roof, sidewalks, etc. snow covered and could not be inspected.  All must meet appropriate codes when completed.

· Interior of garage not available for inspection.  Repair per applicable codes.

This is a registered vacant building.  In order to sell or reoccupy this building, all deficiencies listed on this code compliance report must be corrected within six (6) months of the date of this report.  One (1) six-month time extension may be requested by the owner and will be considered if it can be shown that the code compliance work is proceeding and is more than fifty (50) percent complete in accordance with Legislative Code Section 33.03(f).

You may file an appeal to this notice by contacting the City Clerk's Office at 651-266-8688.  Any appeal must be made in writing within 10 days of this notice.  (You must submit a copy of this notice when you appeal, and pay a filing fee.)

If you have any questions regarding this inspection report, please contact Jim Seeger between 7:30 - 9:00 AM at 651-266-8989 or leave a voice mail message.

Sincerely,

James L. Seeger

Code Compliance Officer

JLS:dj
Attachments
An Equal Opportunity Employer


